Assistive Bottle Opener

3D Printing, Soldering, and Assembly Directions

Author: Amber Cruz

Model Creator: MakersMakingChange (Thingiverse)

Assistive Bottle Opener

3D Printing Instructions

- Download the device file from the virtual library.

- Open PrusaSlicer and click the “Add…” button (see Figure 1), then select the device file from the downloads folder.

Figure 1. Button to Import Files into PrusaSlicer.

- Go to the “Print Settings” tab in the top left corner of PrusaSlicer and change the number of perimeters to 4 (see red arrow in Figure 2). Then double-click on the “Infill” subtab (see blue arrow in Figure 2).

Figure 2. Adjusted Perimeter Settings.

- Change the fill density to 50% and the fill pattern to Gyroid (see red box in Figure 3).

Figure 3. Adjusted Infill Settings.

- Go back to the “Plater” tab in the top left corner of PrusaSlicer. Then click the “Print settings” drop-down menu and select the 0.20mm Structural option (see red arrow in Figure 4).

  - Note: The titles of each print setting option will vary slightly between printer models, so there may not be an option matching this wording exactly – that’s ok! What’s important is that you select one of the 0.20mm options.

Figure 4. Main Plate Settings.

- Open the Filament drop-down menu (see blue arrow in Figure 4) and select the filament type you want to print in.

  - Note: The recommended material for this device is PLA, but PETG is also acceptable.

- Open the Printer drop-down menu (see green arrow in Figure 4) and select your printer make and model.

  - Note: If you don’t see your specific printer in the drop-down menu, follow these instructions to add them: https://help.prusa3d.com/article/configuration-wizard_1754

- Click the “Slice now” button in the bottom right corner of PrusaSlicer and wait for the software to finish slicing the file.

- Once the slicing is finished, click “Export G-code” (see Figure 5) and save the G-code to a flash drive.

Figure 5. Export G-code Button.

- Insert the flash drive into your 3D printer, select the G-code and print!

Assistive Bottle Opener

Soldering Instructions

- No soldering is required for this device.

Assistive Bottle Opener

Assembly Instructions

- No assembly is required for this device.